# Standard Operating Procedure: Coaching Stock Pre-Departure Examination

Document No.: SOP/C&W/PDE/2026/08     Revision: 03     Effective: 01 October 2026

| Field | Details |
| --- | --- |
| Department | Carriage & Wagon (Mechanical) |
| Applicable To | Passenger rakes: LHB and ICF coaches |
| Approved By | Sr. Divisional Mechanical Engineer (fictional) |
| Review Cycle | 12 months |

# 1. Purpose

This SOP defines the mandatory examination of coaching stock before a rake is released for service. It aims to ensure that no rake with a safety-critical defect leaves the primary maintenance depot.

# 2. Scope

The procedure applies to all originating passenger trains with a booked speed above 80 km/h and covers under-gear, bogie, brake system, couplers, doors, electrical and fire-safety items.

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Train Examiner (TXR) | Carry out examination, record defects, issue Fit-to-Run certificate |
| Senior Section Engineer (C&W) | Supervise TXR, approve rectification of Class A defects |
| Loco Pilot | Verify brake continuity test and sign Brake Power Certificate |
| Station Master | Release train only after receipt of Fit-to-Run memo |

# 4. Examination Procedure

## 4.1 Preparation

- Confirm the rake is placed on a pit line with protection flags and a blue flag/light displayed.

- Verify the rake composition and coach numbers against the train consist.

- Check that the previous trip defect memos (if any) have been closed.

## 4.2 Under-gear and Bogie Checks

- Inspect axle boxes for overheating, grease leakage and loose cover bolts.

- Check wheel tread for flats (limit 50 mm length), shelling and thin flange (minimum flange thickness 16 mm).

- Examine primary and secondary suspension springs for cracks or sagging.

- Verify safety straps, bolster anchor links and brake rigging cotter pins are in position.

## 4.3 Brake System Tests

The following tests shall be performed and results recorded in the Brake Power Certificate:

| Test | Parameter | Acceptance Criterion |
| --- | --- | --- |
| Brake pipe (BP) pressure | At engine | 5.0 kg/cm2 |
| Feed pipe (FP) pressure | At engine | 6.0 kg/cm2 |
| BP leakage | 1 minute after isolating | Drop not more than 0.5 kg/cm2 |
| Brake application | BP reduction of 1.0 kg/cm2 | All coaches apply within 30 s |
| Brake release | After full charge | All coaches release within 45 s |
| Brake power | Rake as a whole | Minimum 90% |

## 4.4 Coupling and Gangway

- CBC (Centre Buffer Coupler) height 1105 mm +/- 10 mm above rail level.

- Verify anti-climbers and gangway rubber bellows are intact.

# 5. Defect Classification

| Class | Description | Action |
| --- | --- | --- |
| A | Safety-critical (e.g. cracked axle, brake failure) | Coach detached, no dispensation |
| B | Serious but not immediately unsafe (e.g. loose bolt, minor leakage) | Rectify before departure |
| C | Passenger amenity (e.g. fan, light, toilet tap) | Rectify if time permits, else record |

# 6. Records

The TXR shall retain the Fit-to-Run certificate, Brake Power Certificate and defect register for a minimum of 12 months. All entries shall be digitised in the Coaching Maintenance Management System within 4 hours of departure.

# 7. Revision History

| Rev. | Date | Description |
| --- | --- | --- |
| 01 | 15-Jan-2022 | Initial issue |
| 02 | 10-Feb-2024 | Added LHB coach criteria |
| 03 | 01-Oct-2026 | Updated brake power threshold and records retention |