# Incident Investigation Report: Signal Passed At Danger (SPAD)

Report No.: SAFETY/INV/2026/033     Classification: Consequential Train Accident (fictional scenario)     Status: Final

| Item | Details |
| --- | --- |
| Date and Time | 03 August 2026, 05:42 hrs |
| Location | Km 214/6-8, between Station A and Station B (fictional) |
| Train | 12345 Express, 18 coaches, hauled by electric loco |
| Section | Double line, automatic block, 110 km/h |
| Weather | Dense fog, visibility approx. 60 m |
| Casualties | No fatalities; 3 minor injuries |
| Damage | Estimated INR 18.5 lakh (track and signalling) |

# 1. Executive Summary

The train passed automatic signal AS-214 at danger and came to a stop 410 m beyond the signal, after emergency braking, short of the preceding goods train standing on the same line. A rear-end collision was avoided; however, one coach derailed at a trailing crossover. The investigation concluded that the primary cause was loss of signal sighting in dense fog, combined with a non-functional train protection warning on the locomotive.

# 2. Sequence of Events

| Time | Event |
| --- | --- |
| 05:31 | Goods train 54321 stops at AS-214 (showing red) due to preceding section occupancy |
| 05:38 | 12345 Express passes AS-212 showing double yellow at 98 km/h |
| 05:40 | Express passes AS-213 showing single yellow at 92 km/h |
| 05:41:40 | Loco Pilot sights AS-214 red at approx. 70 m, applies emergency brake |
| 05:42 | Train passes AS-214 at danger at 41 km/h and stops 410 m beyond |
| 05:43 | Loco Pilot informs Section Controller by walkie-talkie |
| 05:49 | Traffic and power block imposed; relief arranged |

# 3. Findings

## 3.1 Human Factors

- The Loco Pilot had completed 7 hours 20 minutes of duty at the time of the incident, within permissible limits.

- The Loco Pilot did not reduce speed adequately after sighting the double-yellow aspect in poor visibility.

- The Assistant Loco Pilot was not calling out signal aspects as required.

## 3.2 Equipment

- The onboard train protection warning system (TPWS) was found isolated by the crew earlier in the journey after repeated spurious alarms.

- Signal AS-214 and its approach aspects were tested and found working correctly.

- Brake power tested at 94% at the originating station; emergency braking distance was consistent with the loaded train weight.

## 3.3 Infrastructure

- Signal sighting distance in clear conditions measured at 420 m, compliant with the standard.

- Fog signal posts were not provided in this block section.

# 4. Root Cause Analysis

| Category | Cause | Contribution |
| --- | --- | --- |
| Primary | Failure to control speed on caution aspects in dense fog | High |
| Contributory | TPWS isolated without authority | High |
| Contributory | Absence of fog-safe device on locomotive | Medium |
| Contributory | Lack of aspect call-outs by Assistant Loco Pilot | Medium |

# 5. Recommendations

- Reinforce fog-season operating instructions: maximum 60 km/h in visibility below 100 m.

- Fit all locomotives with a GPS-based fog safety device and verify before departure.

- Make TPWS isolation subject to controller approval, with automatic event logging.

- Provide fog signal posts at intervals of 1 km on sections with automatic signalling.

- Conduct counselling and retraining of the crew on signal call-out discipline.

# 6. Compliance Tracking

| Rec. No. | Responsible | Target Date | Status |
| --- | --- | --- | --- |
| 1 | Divisional Operations Manager | 30-Sep-2026 | Completed |
| 2 | Chief Electrical Loco Engineer | 31-Dec-2026 | In progress |
| 3 | Chief Signal & Telecom Engineer | 31-Oct-2026 | In progress |
| 4 | Chief Operations Manager | 31-Mar-2027 | Planned |
| 5 | Divisional Operations Manager | 15-Sep-2026 | Completed |